Exercise – Convolution

Question-1

Given the following image F and filter H, answer the following questions:

- Show the result of H*F, given that

F=1245525212143667       H=0101-41010

- A 3X3 median filter is a filter where the output image is obtained by selecting the median of the pixel values under the filter window. Show the result of convolving the median filter with F.

F=1245525212143667

- Comment the property of median filter in comparison to mean filter learnt in class.

Mean filter: 19×111111111

Question-2

K is a 2D symmetric filter of size k×k, and k is an odd number. By convoluting K on an image I (denoted as K*I), we obtain a filtered image IK.

- Given an image pixel at location (x,y), find its value in the filter image IK(x,y). Express your answer in terms of I, K, k, x and y.

- Calculate the number of operations in (a)? An operation is defined as an addition or multiplication of two numbers. Assuming the size of an image is N×N. Express your answer in terms of N and k.

- If K=gh, where g∈Rk×1 and h∈R1×k are two 1D filters, prove that convolution can be performed by (I*g)*h. What is the number of operations required?

- Is (c) more computationally efficient than (b)? How many operations are saved?